| # Material Safety Data Sheet | U.S. Department of Labor |
| --- | --- |
| May be used to comply with | Occupational Safety and Health Administration |
| OSHA's Hazard Communication Standard, | (Non-Mandatory Form) |
| 29 CFR 1910.1200. Standard must be | Form Approved |
| Consulted for specific requirements. | OMB No. 1218-0072 |

| IDENTITY (As Used on Label and List) ## TA0500/ Crowning Glory Clear | Note: Blank spaces are not permitted. If any item is not applicable, or no information is available, the space must be marked to indicate that. |
| --- | --- |

Section I

| ### Manufacturer's Name Crown Research & Chemical Corp | Emergency Telephone Number 210-225-3369 |
| --- | --- |
| Address (Number, Street, City, State, and ZIP Code) P O BOX 12488 | Telephone Number for Information 210-225-3369 |
| San Antonio, Texas 78212 | ### Date Prepared 6/24/04 |
| U.S.A | Signature of Preparer (optional) |

Section II - Hazard Ingredients/Identity Information

| Hazardous Components (Specific Chemical Identity; Common Name(s)) | OSHA PEL | ACGIH TLV | Other Limits Recommended | %(Optional) |
| --- | --- | --- | --- | --- |

Acrylic Copolymer non-hazardous none none N/A

Individual Residual Monomers non-hazardous none none N/A

Water non-hazardous none none N/A

Section III - Physical/Chemical Characteristics

| ### Boiling Point | 212 F | Specific Gravity (H2O = 1) | >1 |
| --- | --- | --- | --- |
| Vapor Pressure (17mm Hg.) | 68 F | ### Melting Point | 32 F |
| Vapor Density (AIR = 1) | <1 | Evaporation Rate (Butyl Acetate = 1) | <1 |
| Solubility in Water: 100% |  |  |  |
| Appearance and Odor: Clear liquid w/slight acrylic odor |  |  |  |

Section IV - Fire and Explosion Hazard Data

| Flash Point (Method Used) Will not burn | ### Flammable Limits N/A | LEL N/A | UEL N/A |
| --- | --- | --- | --- |
| Extinguishing Media: As appropriate for combustibles in the area. |  |  |  |
| Special Fire Fighting Procedures: wear self-contained breathing apparatus and full protective gear |  |  |  |
| Unusual Fire and Explosion Hazards: material can splatter above 100C/212F. Polymer film can burn |  |  |  |

| (Reproduce locally) | OSHA 174, Sept. 1985 |
| --- | --- |

Section V - Reactivity Data

| Stability: | Unstable |  | Conditions to Avoid: none known |
| --- | --- | --- | --- |
| N/A | ### Stable | X | N/A |
| Incompatibility (Materials to Avoid): No known materials which are incompatible |  |  |  |
| Hazardous Decomposition or Byproducts: Thermal decomposition may yield acrylic monomers |  |  |  |
| Hazardous Polymerization | May Occur |  | Conditions to Avoid N/A |
| N/A | Will Not Occur | X | N/A |

Section VI - Health Hazard Data

| Route(s) of Entry: | Inhalation? Slight Irritation | Skin? Slight Irritation | Ingestion? Slight Irritation |
| --- | --- | --- | --- |
| Health Hazards (Acute and Chronic): Any exposure to skin, ingestion or inhalation can result in an Acute irritation. No chronic affects are known. |  |  |  |
| Inhalation of vapor or mist can cause headache-nausea-irritation to nose and throat. |  |  |  |
| Carcinogen: NO | NTP? NO | IARC Monographs? NO | OSHA Regulated? NO |
| N/A |  |  |  |
| Signs and Symptoms of Exposure: slight skin irritation. |  |  |  |
| N/A |  |  |  |
| Medical Conditions Generally Aggravated by Exposure: Watering of eyes, irritation of skin, headaches. |  |  |  |
| Emergency and First Aid Procedures: Wash thoroughly with fresh water for 15 min., if inhaled remove person to fresh air. If irritation persist in any manor call physician. |  |  |  |

Section VII - Precautions for Safe Handling and Use

Steps to Be Taken in Case Material is Released or Spilled:

Evacuate area; Floor may be slippery: use care to avoid falling. Contain large spills immediately with inert materials (e.g. sand, earth).

Keep spills and cleaning runoff out of municipal sewers and open bodies of water.

Waste Disposal Method: Comply with Federal, State And Local regulations. If approved, drain neutralized Product to waste treatment plant, or transfer to waste disposal contractor.

N/A

Precautions to Be taken in Handling and Storing: none required when handling under normal conditions. When storing, keep from freezing and temperatures above 120 F.

Use in ventilated areas only.

Other Precautions: keep container close when not in use. Keep from heat and direct sunlight. Keep from freezing.

Section VIII - Control Measures

| Respiratory Protection (Specify Type): none required under normal conditions. Respiratory mask should be used during spraying or misting operations. |  |  |
| --- | --- | --- |
| Ventilation: | Local Exhaust: | Special: use in ventilated area |
| YES | Mechanical (General): YES | Other: N/A |

| Protective Gloves: none required. Gloves may Provide protection from permeation. | Eye Protection: none required. Use chemical splash goggles |
| --- | --- |
| Other Protective Clothing or Equipment: aprons may be used |  |
| Work/Hygienic Practices: precautionary measures should always be taken when handling chemicals. |  |

| #### Pg 2 of 2 | * U.S.G.P.O.: 1986 - 491 - 529/45775 |
| --- | --- |